Debian installation and setup guide

# Image and installation

Download the latest testing DVD ISO image from http://cdimage.debian.org/cdimage/weekly-builds/amd64/iso-dvd/debian-testing-amd64-DVD-1.iso.

Install VirtualBox from https://www.virtualbox.org/wiki/Downloads.

The following resources should be assigned to the virtual machine:

6-8 GB RAM

80-100 GB HDD

2-4 CPUs (one half of available ones)

- Insert Debian DVD image into the virtual optical drive and start the virtual machine.

- When the DVD boots, select common install.

- Select English for language, Europe/Serbia for location, and United States keymap.

- Name and domain are up to the user (for example hostname: debian, domain: dev)

- Create root password.

- Create your user and its password.

- Use entire disk (guided partitioning), with all files in one partition.

- Don’t scan another DVD.

- Use German network mirror (ftp.de.debian.org).

- No Proxy.

- User may participate in package popularity survey, but it’s not mandatory.

- Select KDE as graphical environment, without printing, web or SSH server. Proceed.

- Install GRUB to the master boot record (/dev/sda).

- Finish the installation.

# Basic system configuration

## sudo

Open a console with Alt+F2 “konsole”.

su

usermod –a –G sudo <username>

Log out and log in for this change to take effect.

## aptitude

First, we must fix the /etc/apt/sources.list file.

sudo nano /etc/apt/sources.list

Comment out DVD line. Add contrib and non-free to the “stretch main” lines.

Ctrl+o to save, Ctrl+x to exit editor.

sudo apt-get update

sudo apt-get install aptitude

## VirtualBox dkms

sudo apti	tude install virtualbox-guest-dkms

This will also install some basic utilities for compiling.

Restart machine.

## Source version control

sudo aptitude install git subversion cvs mercurial bzr darcs

## Editors

sudo aptitude install emacs vim

## Cross-compilation tools

Binutils:

sudo aptitude install binutils-aarch64-linux-gnu binutils-arm-linux-gnueabi binutils-arm-linux-gnueabihf

Gcc:

sudo aptitude install gcc-arm-linux-gnueabi gcc-arm-linux-gnueabihf

## Ncurses

sudo aptitude install libncurses5-dev

## Miscellaneous

sudo aptitude install bc

# Useful packages